CANOPY AI X THERMONET 5G

ASEAN GeoAI Fusion 2026 Competition | A GeoAI planning platform for cooler, more resilient 5G infrastructure

1. Executive Summary

Urban heat is increasingly a network-planning problem. Hot surfaces, dense built environments and cooling demand can raise thermal stress around telecommunications equipment, yet city planners and network operators often lack a single view that connects heat exposure, spatial context and intervention options.

Canopy AI X ThermoNet 5G is a transparent GeoAI planning prototype that turns satellite, urban-form and mobile-network signals into an interactive micro-cooling simulator. It helps stakeholders identify potentially stressed sites, test canopy and cool-roof scenarios, and communicate likely temperature and operating-cost implications before committing to field interventions.

2. The Core Problem

The network's blind spot: Environmental heat is typically managed as a facilities issue, while network planning focuses on coverage and performance. The result is a fragmented picture of how place-based heat conditions can affect equipment stress and cooling requirements.

The planner's blind spot: Satellite temperature, building density and mobile-performance data are available in different formats and tools. Without a shared spatial workflow, these signals are difficult to compare at the level of an individual candidate site.

The investment blind spot: Tree planting and cool roofs are often discussed as broad sustainability measures, not as testable site-level scenarios. Decision-makers need a way to estimate the direction and scale of benefit before prioritising limited budgets.

3. Market Validation & Statistics

•  Prototype node coverage: The active Spring API currently serves four static Kuala Lumpur node records, while the Python preprocessing flow can export 50 processed candidate records for integration testing.

•  Heat-to-energy planning rule: The simulator assumes that each 1 C reduction in site temperature lowers cooling energy use by 3%; this is a disclosed scenario coefficient, not an operational billing guarantee.

•  Thermal signal baseline: Where raster sampling is unavailable, the preprocessing pipeline applies a 35.0 C land-surface-temperature fallback so the demo can continue operating.

•  Model demonstration scale: The current Random Forest is trained on 5,000 synthetic observations to demonstrate the end-to-end modelling pattern before real telemetry is available.

•  Decision range: Thermal Strain Score is normalised to a 0-100 range, enabling quick comparison of current and simulated intervention scenarios.

4. Value Proposition

A. For city planners and sustainability teams

•  Spatial prioritisation: See candidate sites alongside local heat and urban-context indicators to focus field assessment where it is likely to matter most.

•  Scenario storytelling: Use canopy-area and cool-roof controls to explain possible intervention trade-offs in a clear, non-technical interface.

•  Governance-ready evidence: Keep observed data, model outputs and planning assumptions distinct so recommendations are easier to audit and refine.

B. For telecom operators and infrastructure teams

•  Thermal-risk visibility: Bring temperature, density and network-penalty proxies into one site-level planning view.

•  OpEx scenario comparison: Estimate the directional effect of cooling interventions on annual cooling-energy cost using explicit assumptions.

•  Integration pathway: Start with the existing REST dashboard, then connect validated telemetry, PostGIS and live notifications as the operating model matures.

5. Machine Learning / Technical Design

Model Inputs (Features - X)

•  baseLST: Land-surface temperature sampled from the Landsat raster at the candidate tile or node centroid.

•  baseDensity: A current built-environment or proxy density signal; the planned version will derive it from building and spatial-join data.

•  networkPenalty: A load-related proxy derived from mobile performance data in the prototype.

•  Intervention inputs: User-selected canopy area and cool-roof coverage used by the scenario simulator.

Model Outputs (Targets - Y)

•  geoAiProb: A prototype model-inference field representing predicted heat-related risk or likelihood.

•  Thermal Strain Score: A transparent 0-100 scenario score combining current temperature, density and network penalty.

•  Throttling-risk score: A prototype regression output trained on synthetic throttling-risk values. Although the README describes a 72-hour use case, the current code does not implement or validate a forecast horizon.

•  Estimated annual saving: A scenario output based on node power, electricity tariff, cooling dependency and the stated temperature-to-energy coefficient.

6. System Architecture & API Flow

The platform separates spatial computation, API delivery and user-facing simulation. The Python GeoAI tier creates node features; Spring Boot exposes a clean JSON contract; and the React application maps nodes, runs client-side scenario calculations and prepares an executive summary.

•  Load the map: GET /api/v1/nodes - Returns the active processed-node list that React displays at startup.

•  Review the risk example: GET /api/v1/nodes/risk-summary - Returns the current prototype high-risk summary for API demonstration.

•  Run a cooling scenario: Client-side App.jsx calculation - Applies canopy and cool-roof inputs to calculate temperature drop, TSS and estimated savings without modifying observed node data.

•  Create an executive report: Browser PDF generation - Produces a two-page scenario summary for stakeholder discussion.

7. External Data Sources & Integrations

•  Landsat LST raster: Supplies land-surface-temperature values for candidate-node sampling through rasterio.

•  Ookla mobile performance tiles: Provides Parquet tile geometry and average-latency information used to locate candidate nodes and create a network-penalty proxy.

•  Building-height GeoJSON / OSM: Provides the planned source for urban density, built-form and street-canyon features.

•  GeoPandas and pandas: Read, transform and filter vector and tabular spatial data.

•  Spring Boot and Jackson: Serialize processed-node data as REST JSON for the dashboard.

•  React, Leaflet and Recharts: Power the web interface, interactive map and visual reporting components.

8. Execution Plan / Roadmap

Phase 1 - Data and pipeline readiness (GeoAI team)

•  Unify the Python export path with the active API resource or introduce a database-backed handoff.

•  Implement the building-data spatial join and replace the density proxy with documented urban-form features.

•  Add requirements.txt or pyproject.toml and reproducible geospatial environment setup.

Phase 2 - Model validation (Data science and network operations)

•  Replace synthetic training data with governed operator telemetry, weather and historical thermal-alarm records.

•  Measure accuracy, calibration and feature provenance; publish a model card and evaluation report.

•  Validate the throttling-risk model against real throttling or thermal events and define any forecast horizon before operational use.

Phase 3 - Platform integration (Backend and product engineering)

•  Introduce PostgreSQL/PostGIS for nodes, telemetry, scenarios and audit reporting.

•  Configure CORS, profiles, logging, secrets and environment-specific deployment settings.

•  Add API, pipeline and React formula tests to the CI/CD workflow.

Phase 4 - Live decision support (City and operator stakeholders)

•  Connect live ingestion for telemetry, weather and satellite updates.

•  Add WebSocket or SSE updates so the map can refresh node status without a page reload.

•  Pilot prioritised interventions and compare observed outcomes with versioned scenario assumptions.

Planning prototype: scenario outputs are decision-support estimates. They should be field-calibrated and validated before they are used for operational commitments or financial forecasts.

Appendix A. Plain-English Technical Breakdown

This appendix preserves the detailed codebase explanation and has been checked against the current repository. Statements about live data, forecasting and financial savings are explicitly identified as prototype assumptions where applicable.

Plain-English guide to this document

Read this first

This document has two levels. The next pages contain the technical detail: filenames, libraries and formulas. This guide tells you, in simple words, what each big section means and why it matters.

The whole project in one sentence

Canopy AI X ThermoNet 5G is a planning dashboard. It tries to show which 5G locations may be under heat stress and lets a user test whether trees or cool roofs could reduce heat and electricity cost.

1. GeoAI Raster and Spatial Engine

Think of this folder as the research and calculation room. It looks at map data before the dashboard sees anything.

The Landsat TIF file is a heat picture taken from a satellite. Each pixel has a temperature value. The script asks: “What is the satellite surface temperature at this network location?” That answer becomes baseLST.

The Ookla Parquet file is a large table of mobile-network map tiles. The script chooses tiles around Kuala Lumpur, finds the centre of each tile, and uses that centre as a possible network location. In this prototype, average latency is used as a rough sign of network load. It is not a real measurement of equipment heat.

The building GeoJSON is meant to describe nearby buildings. In a mature version it would help measure how built-up an area is. Right now the code does not yet use it, so this is an important next improvement.

preprocess_uhi.py is the “combine everything” script. It turns raw map files into a small node record with coordinates, temperature, density and load-related values. train_model.py is the “practice AI” script. It trains a Random Forest using made-up historical examples so the project can demonstrate where prediction fits.

2. Spring Boot RESTful API Gateway

Think of the API as a waiter between the data and the screen. The React dashboard does not read Python files directly. It asks the Spring Boot API for a clean JSON list of nodes.

NodeController.java is the main waiter. When the UI asks for /api/v1/nodes, it reads processed_nodes.json and sends the node list back. TelecomNode.java is the label sheet that says what each field means: ID, latitude, longitude, temperature, density, load penalty and energy inputs.

Important: there are two processed_nodes.json files. The API uses the one inside 2-api-backend/api/src/main/resources. The Python script currently writes its 50-node output to a different folder, so those 50 records are not what the API serves. This is a wiring issue, not a data-science issue.

3. React Micro-Cooling Simulator

Think of this folder as the user-facing control room. It draws the map, lets a planner pick a location, move two sliders, and download a report.

App.jsx is the main screen. It fetches the API node list, shows the map, and calculates the scenario results. The tree slider adds simulated canopy area. The roof slider adds simulated cool-roof coverage. The screen then shows a lower temperature, a lower strain score and an estimated annual energy saving.

The ASEAN city options outside Kuala Lumpur are named demonstration profiles. They make the interface useful for showing a regional concept, but they are not live operator sites. index.css is only the design layer: it controls the grid, cards, alignment and responsive mobile layout.

4. Full data flow

The intended path is simple: satellite heat image + Ookla tiles + building data -> Python feature calculation -> trained model prediction -> Spring Boot JSON endpoint -> React map and simulator -> executive PDF report.

Today, the project demonstrates each part of that path, but the links are not all live. The API has static JSON, the model is trained on synthetic rows, and the simulator uses clearly stated hard-coded planning coefficients.

5. Thermal score, risk and cost

Thermal Strain Score (TSS) is a simple “how stressed is this site?” number. A hotter surface, denser surroundings and higher network penalty push the score up. Adding simulated cooling pushes it down. The prototype throttling-risk percentage is a prototype indicator, not a proven operational forecast.

The cost figure starts with estimated node power, electricity price and the share of electricity used for cooling. The app assumes that every 1 C temperature reduction saves 3% of cooling energy. This is useful for comparing scenarios, but it is not yet a real utility bill forecast.

CANOPY AI X THERMONET 5G

Technical Architecture and Codebase Breakdown

ASEAN GeoAI Fusion 2026 Competition | Kuala Lumpur GeoAI prototype

# Purpose

This guide maps the actual repository to the intended three-tier workflow: Satellite Raster Extraction -> Spatial Join with Ookla Parquet -> Random Forest Regressor Pipeline -> Spring Boot REST API -> React Micro-Cooling Simulator. It distinguishes the implemented prototype from its planned production architecture.

# Executive architecture

1-geoai-engine  ->  processed_nodes.json  ->  2-api-backend/api  ->  /api/v1/nodes  ->  3-micro-cooling-ui

Raw inputs        feature/model export          REST JSON                dashboard, simulator, PDF summary

## Important prototype boundary

The repository contains real spatial source files, but the current frontend and Spring API serve demo/static node records. The Python training script also generates synthetic training observations. Thermal Strain Score (TSS), prototype throttling-risk, and OpEx figures must therefore be presented as scenario estimates until validated with live operator and field data.

# 1. GeoAI Raster and Spatial Engine

| File/folder | Purpose and technology | Input -> process -> output | Workflow fit |
| --- | --- | --- | --- |
| 1-geoai-engine/ | Python GeoAI tier root | Holds spatial ingestion, feature engineering, model training, data/ and models/ paths. | Inputs raw spatial data; outputs model and node JSON. |
| data/kl_building_heights.geojson | Urban-form source | GeoJSON building footprints/heights intended for OSM-derived density and street-canyon features. | Input to improved density/spatial-join step. |
| data/landsat_lst_kl.TIF | Thermal raster | Landsat land-surface-temperature raster sampled at tile/node centroids using rasterio. | baseLST feature; fallback is 35.0 C if unavailable. |
| data/ookla_performance_mobile_tiles.parquet | Network spatial input | Ookla mobile tiles read with pandas/geopandas; WKT tile converted to geometry; KL bounding box filters tiles. | Centroid becomes node position; avg_lat_ms drives penalty proxy. |
| preprocess_uhi.py | Feature pipeline | Reads Parquet, samples Landsat, derives baseDensity and networkPenalty, optionally runs model inference, writes processed_nodes.json. | Bridge from raw data to Spring-readable node records. |
| train_model.py | Model training | Creates synthetic 5,000-row dataset, trains scikit-learn RandomForestRegressor, writes models/thermal_model.joblib. | Prototype risk-model artifact for preprocess inference. |
| models/ (expected) | Model-artifact folder | Created only after training; not currently present in listed source tree. | Expected dependency of preprocess_uhi.py. |

Methodology tie-in: GeoAI Raster Engine. rasterio provides point sampling; GeoPandas provides vector geometry/CRS operations; pandas reads the Parquet table; joblib persists the model. Current caveat: OSM GeoJSON is named as an input but is not actually read by preprocess_uhi.py; density is currently derived from latency.

# 2. Spring Boot RESTful API Gateway

| File/folder | Purpose and technology | Input -> process -> output | Dependency / caveat |
| --- | --- | --- | --- |
| 2-api-backend/api/ | Maven/Spring Boot module | Buildable API application; mvnw/mvnw.cmd and .mvn wrapper enable consistent Maven execution. | Runs the REST tier on localhost:8080. |
| pom.xml | Build manifest | Java 17, Spring Boot Web MVC, DevTools, Jackson databind and test starter. | Declares API dependencies. |
| ApiApplication.java | Application entry point | SpringApplication.run bootstraps component scanning and embedded server. | Starts the REST gateway. |
| model/TelecomNode.java | JSON domain model | Maps id, coordinates, temperature, density, penalty, power, tariff and cooling dependency. | Contract from JSON to frontend. |
| controller/NodeController.java | Primary endpoint | GET /api/v1/nodes reads classpath processed_nodes.json with Jackson. | Current source for React node list. |
| controller/ThermalRiskController.java | Prototype endpoint | GET /api/v1/nodes/risk-summary returns one hard-coded high-risk sample. | Not consumed by current React UI. |
| resources/application.properties | Spring configuration | Currently only spring.application.name=api. | Place for port/database/logging configuration. |
| api/src/main/resources/processed_nodes.json | Active API data | Four static Kuala Lumpur node records; missing power/tariff fields, so UI uses fallbacks. | Actually served by NodeController. |
| 2-api-backend/src/main/resources/processed_nodes.json | Misplaced generated export | 50 processed records produced by Python output path. | Not on api module classpath; currently unused. |

Methodology tie-in: RESTful API Gateway. The intended design is Python export -> Spring service -> React. The output-path mismatch is the key integration gap: either write directly to api/src/main/resources for the demo, or replace both JSON files with a database for live operations.

# 3. React Micro-Cooling Simulator

| File/folder | Purpose and technology | Input -> process -> output | Workflow fit |
| --- | --- | --- | --- |
| 3-micro-cooling-ui/ | React/Vite application | Client dashboard, ASEAN location selector, map, scenario controls and browser PDF generator. | Consumes /api/v1/nodes. |
| package.json / package-lock.json | npm manifest/lock | React 19, React DOM, Leaflet, React Leaflet, Recharts; Vite and ESLint tooling. | Defines reproducible UI install/build. |
| src/main.jsx | React bootstrap | Mounts App into #root and imports index.css. | Entry point after index.html. |
| src/App.jsx | Main UI and logic | Fetches nodes; supplies ASEAN demo profiles; computes TSS, risk, temperature reduction, cost saving; generates two-page PDF. | Core React Micro-Cooling Simulator. |
| src/index.css | Dashboard styling | Defines responsive header, map/sidebar, cards, metric grids and footer. | Visual hierarchy for simulator. |
| src/App.css | Legacy Vite CSS | Not imported by main.jsx/App.jsx; contains unused starter styles. | Safe cleanup candidate. |
| src/assets/ | Bundled assets | hero.png, react.svg, vite.svg. Current App does not import them. | Optional UI media; some are starter assets. |
| public/favicon.svg, public/icons.svg | Public static assets | Copied unchanged to build output. | Browser branding/static references. |
| index.html | Vite HTML shell | Contains #root mount target. | Feeds main.jsx. |
| vite.config.js | Vite config | Registers React plugin. | Dev server and production build. |
| dist/ | Generated build output | Created by npm run build. | Deployable static frontend; do not hand edit. |
| node_modules/ | Installed dependencies | Generated by npm install. | Local development only; do not commit. |

TSS and scenario logic: currentTemp = baseLST - (canopy_m2 x 0.012 + coolRoof_percent x 0.018). TSS = clamp((currentTemp x baseDensity) + networkPenalty - (temperatureDrop x 2.5), 0, 100). Annual saving = daily cooling cost x (temperatureDrop x 3%) x 365. These coefficients are hard-coded prototype assumptions and are disclosed in the UI/PDF.

# Full data-flow trace

• 1. Landsat LST raster and Ookla Parquet are stored under 1-geoai-engine/data/.

• 2. preprocess_uhi.py reads Ookla tiles, creates GeoDataFrame geometry from WKT, filters the Kuala Lumpur bounding box, and uses the tile centroid as a candidate node location.

• 3. rasterio samples landsat_lst_kl.TIF at each centroid for baseLST; the script derives baseDensity and networkPenalty. The intended next step is a true spatial join with kl_building_heights.geojson.

• 4. train_model.py trains RandomForestRegressor on synthetic feature rows and saves thermal_model.joblib. preprocess_uhi.py loads it when available and adds geoAiProb.

• 5. The pipeline exports processed_nodes.json. In the current repository the exported file is outside the active api module, while NodeController reads api/src/main/resources/processed_nodes.json.

• 6. Spring Boot serializes the active JSON as GET /api/v1/nodes. React fetches it once at startup.

• 7. App.jsx maps nodes, calculates scenario outcomes locally, and produces the executive PDF. The browser-side simulator includes KL API records plus named ASEAN demonstration profiles.

# Production gaps and recommended additions

| Missing/expected item | Why it matters |
| --- | --- |
| models/thermal_model.joblib | Run train_model.py or store a versioned validated model artifact. |
| requirements.txt or pyproject.toml | Pin geopandas, rasterio, pandas, scikit-learn, joblib and spatial dependencies. |
| Dockerfile(s) and docker-compose.yml | Package Python, API, UI and optional PostGIS for portable deployment. |
| application.yml / database configuration | Configure PostgreSQL/PostGIS, profiles, CORS, logging and secrets. |
| Database schema/migrations | Persist nodes, time-stamped telemetry, observations, scenarios and audit reports. |
| Live ingestion service | Accept operator telemetry/weather feeds and replace bundled JSON. |
| WebSocket or SSE endpoint | Push changed node statuses to the React map without refresh. |
| Model evaluation artifacts | Add train/test split report, metrics, feature provenance, calibration and model card. |
| Tests | Add Python pipeline tests, API endpoint tests, and React component/formula tests. |
| CI/CD workflow | Build/lint/test and deploy safely on push. |

# Recommended live architecture

Operator telemetry + weather + satellite updates -> ingestion service -> PostgreSQL/PostGIS -> feature pipeline/model service -> Spring Boot API + SSE/WebSocket -> React map and executive reporting. Keep scenario assumptions versioned and separate from observed data. Validate the throttling-risk model against historical thermal alarms before operational use.

# Prototype positioning for ASEAN GeoAI Fusion 2026

This codebase demonstrates the workflow and decision interface: it makes spatial, thermal and infrastructure indicators understandable to DBKL, MCMC and operator stakeholders. It should be presented as a transparent planning demonstrator, not a production prediction system. Its immediate value is prioritisation, reporting, data-governance design and a clear path to field-calibrated deployment.

附录 B：中文项目说明

这部分用较容易理解的中文说明项目目前真正做到什么、哪些结果只是原型估算，以及各个代码部分如何连接。

一句话理解项目

Canopy AI x ThermoNet 5G 是一个规划原型：它把卫星地表温度、移动网络地图资料和简单的机器学习流程放在一起，帮助使用者比较“增加树冠”或“使用冷屋顶”后，某个 5G 候选地点的温度、热压力分数和节能估算可能如何变化。

1. 这个项目要解决什么问题

城市里的 5G 设备可能受到高温、周围建筑密度和网络负载的共同影响。现实中，城市绿化团队、设施团队和电信团队通常各自使用不同数据，因此不容易在同一张地图上讨论哪里应该优先做实地检查或降温措施。

本项目不是已经上线的故障预测系统，而是把这些讨论变成可视化情境分析的工具。它的价值是帮助团队先排序、沟通和设计下一步的数据收集方式。

2. Python GeoAI 引擎做什么

•  Landsat 温度栅格：把卫星影像看成很多小格子，每个格子都有地表温度。程序会在候选位置抽取该温度，作为 baseLST。

•  Ookla Parquet：这是移动网络地图格网数据。程序把格网中心点当成候选位置，并把平均延迟暂时当作网络负载的代理指标。延迟并不等于设备真实温度，因此这是原型假设。

•  建筑 GeoJSON：文件已经放在资料夹中，原本用于计算建筑密度和街谷效应；但目前的 preprocess_uhi.py 还没有读取它。

•  机器学习模型：train_model.py 生成 5,000 笔模拟训练资料，再训练 Random Forest。输出是模拟的 throttling-risk 分数，不是经过真实营运事件验证的预测。

3. REST API 和前端如何连接

REST API 可以简单理解成前端向后端“拿资料”的通道。React 前端启动时会呼叫 GET /api/v1/nodes；Spring Boot 再从它自己的 resources 资料夹读取 processed_nodes.json，并以 JSON 格式回传。

目前真正被 API 使用的是 2-api-backend/api/src/main/resources/processed_nodes.json 中的 4 笔静态记录。你打开的 2-api-backend/src/main/resources/processed_nodes.json 虽然有 50 笔处理结果，但目前不在 API 模组的 classpath 内，所以前端不会使用它。

4. React 降温模拟器如何计算

•  树冠和冷屋顶：滑杆分别输入树冠面积和冷屋顶覆盖率。前端用写死的系数计算温度下降：树冠面积 × 0.012，加上冷屋顶百分比 × 0.018。

•  热压力分数 TSS：TSS 把当前温度、密度和网络 penalty 组合起来，并限制在 0 至 100。分数适合比较同一原型中的不同方案，不应当解释为真实设备健康指数。

•  节能金额：前端以节点功率、电价、冷却依赖度和“每下降 1 C，冷却能源下降 3%”的假设估算年度节省。这不是实际电费预测，需要本地校准。

5. 目前哪些资料是真的，哪些是原型

•  真实或本地资料：资料夹中包含 Landsat TIF、Ookla Parquet 和建筑 GeoJSON；Python 程序也确实会读取 Ookla，并尝试读取 Landsat。

•  静态或模拟资料：API 当前服务 4 笔静态节点；机器学习训练资料是合成的；非吉隆坡城市档案和部分情境系数也在前端写死。

•  尚未完成：建筑数据空间连接、实时营运遥测、数据库、模型评估、实时地图更新和经过实地验证的温度/节能关系。

6. 展示时可以怎样说明

“这是一个透明的 GeoAI 规划原型。它展示从空间资料、特征处理、REST API 到可视化情境模拟的完整流程。下一阶段会以真实设备遥测、经过校准的降温证据和历史热事件来验证模型与节能假设。”

附录 C：中文技术与代码结构详解

以下说明对应仓库中的实际文件，帮助你在展示或答辩时解释每个程式档案的职责。

C1. 整体代码结构

仓库分成三个主要部分：1-geoai-engine 负责 Python 空间资料处理；2-api-backend/api 是 Spring Boot API 模组；3-micro-cooling-ui 是 React 网页介面。预期流程是“原始空间资料 → Python 特征处理/模型 → JSON → Spring Boot → React 地图与模拟器”。

当前版本中，最后两层已经连通：React 会向 Spring Boot 取得节点 JSON。Python 处理结果写入另一个 resources 路径，因此该 50 笔 JSON 目前没有被运行中的 API 使用；运行中的 API 仍读取 api 模组内的 4 笔静态记录。

C2. GeoAI 引擎：1-geoai-engine

•  data/landsat_lst_kl.TIF：吉隆坡地表温度栅格。rasterio 会用候选节点的经纬度从影像抽样，得到 baseLST。若影像无法开启，主流程使用 35.0 C 作为备用值；若抽样发生异常，函数内部会回传 32.0 C。

•  data/ookla_performance_mobile_tiles.parquet：移动网络表现格网资料。preprocess_uhi.py 先以 pandas 读取，再把 tile 的 WKT 文字转换为 GeoPandas 几何；接着以吉隆坡 bounding box 筛选，并最多取 50 个格网。

•  data/kl_building_heights.geojson：建筑高度/建筑物资料。程式已有 OSM_GEOJSON 路径常数，但当前 preprocess_uhi.py 没有读取或空间连接此档案；因此 baseDensity 目前不是由建筑数据真正计算。

•  preprocess_uhi.py：核心预处理档。它对每一个 Ookla tile 取 centroid，抽取温度，并用 1.2 + avg_lat_ms/100 产生 baseDensity，用 avg_lat_ms 产生 networkPenalty；然后组成每个节点的 JSON。

•  节点 JSON 字段：每笔资料包括 id、lat、lng、location、baseLST、baseDensity、networkPenalty、basePowerW、tnbRateKwh、coolingDependency 与 geoAiProb。basePowerW、电价和冷却依赖度都是原型计算输入。

•  train_model.py：产生 5,000 笔随机模拟资料，以 baseLST、baseDensity、networkPenalty 为 X，以 throttling_risk 为 Y，训练 RandomForestRegressor（100 棵树、最大深度 10），并将模型存成 models/thermal_model.joblib。

•  模型载入行为：preprocess_uhi.py 会尝试读取 thermal_model.joblib；若找不到，geoAiProb 会使用 50.0 的备用值。因此目前看到的 50.0 并不代表真实机器学习预测。

C3. Spring Boot API：2-api-backend/api

•  ApiApplication.java：Spring Boot 的启动点。运行 Maven/Spring Boot 后，嵌入式服务器会启动，默认可由 React 在 localhost:8080 存取。

•  NodeController.java：定义 GET /api/v1/nodes。它使用 Jackson ObjectMapper 读取 classpath 中的 processed_nodes.json，再转换为 TelecomNode 清单回传。React 实际调用的就是这个端点。

•  ThermalRiskController.java：定义 GET /api/v1/nodes/risk-summary。这个端点回传一笔写死的高风险示例（包含 88.5 分与 HIGH_THROTTLING_RISK）；当前 React 介面没有呼叫它。

•  TelecomNode.java：Java 资料模型，定义 API 可以读取和回传的字段，例如座标、基础温度、密度、network penalty、功率、电价与冷却依赖度。

•  api/src/main/resources/processed_nodes.json：真正由 NodeController 读取的档案。目前有 4 笔静态吉隆坡节点记录，所以这 4 笔才是地图初次载入时显示的数据。

•  src/main/resources/processed_nodes.json：Python 当前写入的 50 笔档案，位于 API Maven 模组以外；它不会被 NodeController 的 classpath 读取。因此它保留作未接入的处理输出。

C4. React 前端：3-micro-cooling-ui

•  src/main.jsx：React 入口。它把 App 元件挂载到 index.html 的 #root，并引入 index.css。

•  src/App.jsx：主要的页面和业务逻辑。useEffect 会 fetch http://localhost:8080/api/v1/nodes；成功时使用 API 节点，失败时使用各城市写死的 demoNodes。

•  CITY_OPTIONS 与 CITY_NODE_PROFILES：定义吉隆坡、新加坡、雅加达等城市的演示档案。这些不是实时运营商节点，而是为了展示区域化介面而写入前端的模拟数据。

•  Leaflet 地图：MapContainer 和 CircleMarker 把节点画在地图上。用户点击圆点后，右侧面板显示目前条件和降温滑杆。

•  index.css：只负责视觉设计，例如标题、地图与侧栏布局、卡片、指标格和移动装置响应式排列；它不处理数据或公式。

C5. 前端公式逐项解释

•  温度下降：temperatureDrop = canopy × 0.012 + roof × 0.018。canopy 的单位是平方米，roof 是百分比。两个系数写死在 App.jsx，应被视为展示用情境参数。

•  当前温度：currentTemp = baseLST - temperatureDrop。它表示用户设定降温措施后的模拟地表温度。

•  基础热压力：baselineStrain = baseLST × baseDensity + networkPenalty。它是未采取措施前的内部比较值。

•  TSS：strain = clamp((currentTemp × baseDensity) + networkPenalty - temperatureDrop × 2.5, 0, 100)。clamp 表示最后强制限制在 0 至 100。

•  节能估算：dailyCoolingCost = (powerW/1000) × 24 × tariff × coolingDependency；annualSavings = dailyCoolingCost × (temperatureDrop × 0.03) × 365。页面也明确写出“每降低 1 C 节省 3% 冷却能源”的假设。

•  风险显示：若节点有 geoAiProb，前端会直接显示该值；没有时则用 baselineStrain × 0.82 计算一个备用分数。此逻辑不是经过验证的实际故障预警。

C6. 当前限制与下一步

要把原型提升为真实决策系统，优先事项是：把 Python 输出接入 API 模组或数据库；用建筑 GeoJSON 真正计算城市形态特征；以真实设备遥测和历史事件替换合成训练资料；验证模型准确度及降温/节能系数；再加入 PostgreSQL/PostGIS、测试、版本控制和实时更新。